# Task 1:

# Task 2:

#include <msp430g2553.h>

void main(void)

{

unsigned char a = 0xAB;

unsigned char b = 0xFE;

unsigned char sum = a + b;

unsigned char not = ~a;

unsigned char xor = a^b;

unsigned char rightshift = a >> 4;

unsigned char leftshift = b << 2;

a ^= BIT3;

b ^= BIT7;

}

# Task 3:

#include <msp430g2553.h>

#include <string.h>

char* ToUpper(char* Str);

void main(void)

{

char a[] = "1a2b3c#&/%)/&/€#369+'-k";

char* Capital = ToUpper(a);

printf("%s \n", Capital);

}

char* ToUpper(char* Str) {

/*

Stora A = 0x41

Lilla a = 0x61

Funktionen blir då c + 0x20;

*/

int length = strlen(Str);

printf("%d \n", length);

int i;

for(i = 0; i < length; i++) {

if(Str[i] >= 0x61 && Str[i] <= 0x7A) {

Str[i] = Str[i] - 0x20;

}

}

return Str;

}

# Task 5:

#include <msp430g2553.h>

void main(void)

{

// not needed to care about

WDTCTL = WDTPW + WDTHOLD; // Stop watchdog timer

// Change the pin values

P1DIR |= 0x01 + 0x04 + 0x40; // Set P1.0, P1.1 and P1.6 to output direction

// p1.4 --> p1.5

P1IE |= 0x20; // P1.4 interrupt enabled

P1IES &= ~0x20; // P1.4 rising edge

P1IFG &= ~0x20; // P1.4 IFG cleared interrupt flag cleared

//P1OUT &= 0xFD; //1111 1101 //pull low

_BIS_SR(GIE); // Global Interrupt Enabled

// useless loop to make delay

while(1)

{

int a = 0;

int b = 0;

for(; a< 20000; a++)

{

for(;b < 20000; b++)

{

_NOP();

}

}

P1OUT ^= 0x04; //00000010

P1OUT ^= 0x40;//0100 0000

}

}

// Port 1 interrupt service routine

#pragma vector=PORT1_VECTOR

__interrupt void Port_1(void)

{

P1OUT ^= 0x01; // P1.0 = toggle

// change to 1.5

P1IFG &= ~0x20; // P1.4 IFG cleared

}

# Task 6:

#include<msp430g2553.h>

void main(void) {

WDTCTL = WDTPW + WDTHOLD;

// GPIO konfig

P1DIR |= BIT0; // LED1

P1DIR |= BIT6; // LED 6

P1DIR &= ~BIT3; //Input = 0

// Initialstate LEDs

P1OUT |= BIT0; // Red LED on

P1OUT &= ~BIT6; // Green LED off

// Pullup resistor

P1REN |= BIT3; // Enable resistor

P1OUT |= BIT3; // Enable pullup

// Interrupts

P1IE |= BIT3; // Enable interrupt

P1IES |= BIT3; // Fallingedge interrupt

P1IFG &= ~BIT3; // Clear the flags

while(1) {

__bis_SR_register(GIE + LPM4_bits);

}

}

#pragma vector = PORT1_VECTOR

__interrupt void btn(void) {

if(P1IES & BIT3) { // Check which edge triggered

P1OUT |= BIT6; // Green LED on

P1OUT &= ~BIT0; // Red LED off

P1IES &= ~BIT3; // Risingedge interrupt

} else {

P1OUT |= BIT0; // Red LED on

P1OUT &= ~BIT6; // Green LED off

P1IES |= BIT3; // Fallingedge interrupt

}

P1IFG &= ~BIT3;

}